# USER MANUAL:

# AUTOMATED SHOE POLISHING MACHINE

Designed and Developed By: Ajibowu Abdulbasit Olalekan 200211017

Supervised By: Prof. A.I.O Yussuff

Lagos State University, Electronic and Computer Engineering

1. Introduction

This manual provides detailed instructions for the safe and effective operation of the Automated Shoe Polishing Machine. The machine is designed to efficiently clean and polish formal leather shoes (loafers).

1.1 System Overview

The system is controlled by an Arduino Uno Microcontroller, which manages the DC motors for brush rotation, the NEMA 17 Stepper Motor for precise shoe rotation and brush translation, and a mini diaphragm pump for liquid polish dispensing.

//system assembly img

## 2. Safety and General Information

| ## Category | ## Safety Precaution |
| --- | --- |
| ## Power & Electrical | ## The machine operates using a 12V DC adapter. Do not expose the machine to water or attempt to operate it with wet hands. Always unplug the unit before performing maintenance. |
| ## Shoe Compatibility | ## This machine is specifically designed for formal leather cover shoes. Do not insert boots, high heels, or casual shoes, as they may cause mechanical jamming or damage the brushes. |
| ## Mechanical Safety | ## Keep fingers, hair, and clothing away from the rotating brushes and the shoe rotating mechanism during operation. Ensure the brush path is clear before starting the cycle. |

## 3. Machine Overview and Status Indicators

The system communicates its status through an LCD Display and LED Indicators located on the front panel.

| LED Indicator | Color | LCD Display | System State | Audio Feedback |
| --- | --- | --- | --- | --- |
| Power | Red | - | Power inserted into the machine | - |
| System Active | Yellow | Shoe Polishing Machine | System is powered on, initialized, and awaiting color selection. | - |
| - | - | Select Color | Waiting for user input | - |
| - | - | Cleaning Shoe | Cleaning cycle active | - |
| - | - | Polishing Shoe | Polishing cycle active | - |
| - | - | Polishing Complete | The polishing cycle is finished and the shoe is ready for removal. | Alarm |

### 4. System Components (Internal View)

The entire operation is controlled by the Arduino Uno Microcontroller8. The DC Motors power the rotating brushes, and the NEMA 17 Stepper Motor precisely handles the shoe rotation. Polish is dispensed via a mini diaphragm pump.

//internal img of machine,,,

## 5. Step-by-Step Operation Guide

Follow this sequence to operate the Automated Shoe Polishing Machine, as defined by the system flowchart.

| Step | Action | User Input | Machine Response & Display |
| --- | --- | --- | --- |
| 1. Power On | Plug the 12Vold DC adapter into the power socket. | None | LCD: Displays welcome message: "Shoe Polishing Machine". LED: Green LED turns ON (Ready). |
| 2. Shoe Placement | Place your leather shoe (heel first) onto the shoe holder mechanism. | None | Ensure the shoe is securely positioned for the NEMA 17 motor to rotate it. |
| 3. Color Selection | Press the button corresponding to your desired polish color. | Press BLACK or BROWN button13. | LCD: Displays the selected color (e.g., "Color: Black Selected"). |
| 4. Start Cycle | Confirm the shoe is properly secured and the correct color is selected. | Press the START button. | LED: Blue LED turns ON (Polishing). The polishing sequence begins. |
| 5. Polishing Cycle | Keep hands clear of the machine. | None | Brushes: DC Motors activate to rotate brushes. Dispenser: Pump activates to dispense liquid polish. The cycle runs for a predetermined period. |
| 6. Cycle Complete | Wait for the audio notification. | None | Motors/Pump: Deactivate. Buzzer: Alarms completion. LCD: Remove Shoe. |
| 7. Reset | Remove the polished shoe from the holder. | None | The system automatically resets to the initial state, ready for the next shoe. |

## 6. Maintenance Routine

| ## Category | ## Safety Precaution |
| --- | --- |
| ### Polish Refill | ## Regularly check the liquid polish reservoir for both colors. Use only high-quality, non-clogging liquid polish. |
| ## Nozzle Cleaning | ## Periodically clean the dispensing nozzle openings to prevent blockage. |
| ## Input voltage |  |